SDR-Based Carrier Monitoring & Interference Detection System

1. Project Title & Description

Carrier Monitoring System A high-performance spectrum analysis and signal monitoring suite designed for Raspberry Pi and Software-Defined Radio (SDR) hardware. This system provides real-time detection of authorized and unauthorized transmissions by combining GNU Radio signal processing with a custom Python-based visualization and management layer.

2. Purpose

In complex RF environments, identifying unauthorized signals or "rogue" carriers is critical for maintaining link integrity. This project automates the process of spectrum surveying, comparing live signals against a database of "authorized" frequencies to immediately flag interference or unexpected spectrum usage.

3. Features

- Real-Time Carrier Detection: Automatically identifies signal peaks, calculates their center frequency, bandwidth, and power.

- Authorization Management: A built-in Web UI to manage a list of "Known Good" frequencies at runtime.

- Interference Alerting: Visual and logged alerts when a signal is detected outside of the authorized parameters.

- Distributed Architecture: Uses ZeroMQ (ZMQ) to decouple heavy signal processing (GNU Radio) from the visualization and logging dashboard.

- Dynamic Configuration: Adjust FFT size, sample rates, and detection thresholds without restarting the flowgraph.

4. Technologies Used

- Languages: Python 3

- Signal Processing: GNU Radio, SciPy, NumPy

- Communication: ZeroMQ (ZMQ), XML-RPC

- Web Framework: Flask (for the Authorization Manager)

- GUI: PyQt5, Matplotlib

5. Project Structure

- sdr_scipy.grc / .py: The core GNU Radio flowgraph that interfaces with the SDR hardware and publishes PSD data.

- Interference.py: The main visualization dashboard and interference detection logic.

- config_manager.py: A Flask-based utility to manage authorized frequency ranges via a web browser.

- sdr_scipy_epy_block_1.py: Embedded Python block for real-time carrier clustering and power estimation.

6. Usage

Prerequisites

Ensure you have GNU Radio 3.10+ and the required Python libraries installed:

Bash

pip install flask pyzmq numpy scipy matplotlib PyQt5

Running the System

The system operates in three parts that should be started in order:

- Start the Authorization Manager: This opens the Web UI at http://localhost:5580 to define your "Authorized" signals.

Bash

python3 config_manager.py

- Run the GNU Radio Flowgraph: This starts the SDR data acquisition and ZMQ transmission.

Bash

python3 sdr_scipy.py

- Launch the Interference Dashboard: This displays the spectrum and highlights unauthorized carriers in real-time.

Bash

python3 Interference.py

Example: Authorizing a Signal

- Open http://localhost:5580 in your browser.

- Add a new frequency:

  - Label: "Satellite Uplink A"

  - Center: 70000000 (70 MHz)

  - Bandwidth: 1000000 (1 MHz)

- The Interference.py dashboard will now treat any signal within 69.5–70.5 MHz as "Authorized." Any signal appearing outside this range will be flagged in the logs.

#PROMPT

System Overhaul: Mission-Critical Live Monitoring Platform

PROJECT TITLE: ISTRAC Carrier Monitoring System (CMS) – Multi-Node SDR Surveillance Platform

PROJECT TYPE: High-security, government-grade mission control dashboard. The system utilizes a distributed architecture where multiple Raspberry Pi + SDR nodes perform edge-processing (interference.py) and stream real-time telemetry to a central dashboard.

CORE ARCHITECTURE & BEHAVIOR:

- Centralized Command: A single dashboard server manages all node connections.

- Decoupled Views (CRITICAL):

  - "View Details" Action: Opens a dedicated asset page showing satellite specs, station location, and hardware ID. No live data here—only technical documentation and static parameters.

  - "Open Spectrum Monitor" Action: Switches to the High-Intensity Telemetry View. This loads the live stream from the specific Node IP (e.g., http://192.168.1.50:PORT).

- No Dataset Uploads: System is strictly 100% Real-Time Live Monitoring.

1. SECURE GATEWAY (Login Page)

- Visuals: Dark mission-control aesthetic. Background: Earth from Orbit (low-opacity). Centered, semi-transparent frosted-glass login card.

- Heading: ISRO – ISTRAC | Carrier Monitoring System.

- Auth: Login field (admin@istrac.gov.in), toggleable password field, and "Secure Session" persistence.

2. CENTRAL MISSION HUB (Main Dashboard)

- Header: Centered "Carrier Monitoring System". Top-right: Live UTC Date & Time (HH:MM:SS, auto-updating).

- Sidebar: Multi-antenna quick-switch sidebar for rapid jumping between nodes.

- Antenna Grid: * Cards for GSAT-30, INSAT-25, GISAT-1, GSAT-24, CMS-02.

  - Live Status: Pulsing GREEN (Online/Heartbeat detected) or Static RED (Node Offline).

  - Navigation: Two distinct buttons: [VIEW DETAILS] and [OPEN SPECTRUM MONITOR].

3. COMPONENT A: ANTENNA DETAIL VIEW (Static Asset Data)

- Layout: Clean, document-style technical layout.

- Specs: Satellite Name, Lat/Long, Band (C/Ku/Ka), Operating Freq Range, Allocated Bandwidth.

- Hardware: SDR Type (HackRF/RTL), Raspberry Pi Node ID, Station Location.

- Imagery: High-res satellite render and Ground Station site photo placeholder.

4. COMPONENT B: LIVE SPECTRUM MONITOR (Dynamic Mission Data)

- Live Stream: Embedded real-time spectrum plot sourced from the Node's interference.py.

- Visual Indicators:

  - Authorized Carriers: Highlighted with green transparent overlays.

  - Unauthorized Signals: Highlighted in red with blinking "INTERFERENCE" tags.

  - Noise Floor: Dynamic baseline visualization.

- Telemetry Logs: A scrolling "Live Event Log" on the right side showing timestamped carrier detections (e.g., [14:20:05] NEW CARRIER DETECTED: 4120.5 MHz).

- Node Heartbeat: A "Node Health" meter showing CPU temp of the Pi and SDR signal-to-noise ratio.

- Action Bar: [SNAPSHOT SPECTRUM], [EXPORT DETECTION REPORT], [REFRESH RATE SETTINGS].

5. GEOSPATIAL NODE MAP (Leaflet.js)

- Theme: Dark-mode satellite map.

- Markers: Interactive markers at station locations (Hassan, Sriharikota, etc.).

- Interaction: Clicking a marker triggers a "Quick-Action" popup: Details or Monitor.

6. AUTHORIZED FREQUENCY MANAGER (Config Integration)

- Function: Direct interface for config_manager.py.

- Actions: Add/Delete frequencies from the "Allowed" list.

- Real-time Push: Changes saved here must trigger an immediate update to the interference.py detection mask on the respective Raspberry Pi node.

TECHNICAL & VISUAL REQUIREMENTS:

- Theme: Professional Dark Mode (Space Operations Center style).

- Color Palette: #0B0E14 (Deep Space), #00E5FF (Cyan Telemetry), #FF3D00 (Alert Red).

- UI Standards: Clean typography (technical/sans-serif), high-density data without clutter, responsive LAN-wide access.

- Advanced Features: Last detection timestamp, unauthorized carrier alert counter, and a global notification panel for system-wide interference alerts.

FINAL INSTRUCTION: Build this as a high-performance mission operation platform. Prioritize the separation of Details (Info) vs Monitor (Live Action) to ensure the operator can focus on signal surveillance without distraction.