- Hardware Sample Rate is the ADC sampling rate of the SDR front-end. If: HW_SAMPLE_RATE = 20e6 Then the SDR captures: ±10 MHz around the hardware centre frequency.

- Display Bandwidth is purely a visual scaling parameter. It defines: X-axis span in the GUI, Frequency labelling limits and Zoom window for plotting. It does not change: Actual sampled data, FFT bin spacing, Detection logic. If DISPLAY_BW = 20e6 You are just telling Matplotlib, Plot ±10 MHz on the x-axis, even if hardware sampled 20 MHz or 40 MHz

DISPLAY_BW != HW_SAMPLE_RATE

- Hardware Centre Frequency is the actual LO tuning frequency of the SDR i.e. SDR LO tuned to 70 MHz Spectrum spans 70MHz±10 MHz. It Display Centre Frequency is  a visual offset only You want to shift spectrum in GUI, You are digitally frequency shifting data, You want logical relabelling. It does NOT retune the SDR.

- MORPH_MIN_WINDOW_BINS  = 101   it acts like a sliding vacuum cleaner that sucks away all the tall signal mountains and leaves behind only the flat ground (the noise floor). Basically this parameter keeps the noise floor line at the very bottom (underneath the carriers), the code can accurately measure if the entire floor moves up. If the line rises by 5dB, the code knows it’s not just a loud carrier—it’s a global problem like interference or a hardware fault. 101 because, it can see the entire carrier rather than the parts.

- MORPH_SMOOTH_WINDOW  is a hard-coded constant used inside the estimate_local_noise_floor function. It is part of the mathematical "back-end" that calculates the noise floor. This window of 51 bins smooths those jagged steps into a clean, natural-looking line.

Anamoly detection

The "Global Noise Rise" warning is a Relative Comparison between the past and the present.

The Variables Involved:

- noise: The current median noise level of the current frame.

- _noise_history: An array (memory) that stores the noise levels of the last 100 frames.

- historical_baseline: The median value of that history (the "normal" state).

- ANOMALY_THRESHOLD_DB: The trigger limit (set to 5.0 dB).

- rise_db: The calculated difference (Current Noise - History).

The code keeps a running record of the background noise. It constantly asks: "Is the noise I see right now significantly higher than what I've seen over the last few seconds?"

- If rise_db is greater than 5.0 dB, the system flags an Anomaly.

- It doesn't matter if a carrier is loud; because of the Morphological filter (which "sucks away" carriers), the system is only looking at the empty space between signals. If that empty space gets louder, it knows there is a global problem.

This logic detects:

- LNA saturation

- Strong wideband jammer

- Adjacent satellite interference

- Rain fade noise rise

- Receiver compression

- AGC malfunction

- RF chain instability

- Temperature drift

- Sudden spectrum congestion

Carrier Detection and highlight