Input:

Freq_segments (bins): 70.000 70.005 70.010 70.015 70.020 70.025 70.030

psd_segment

- Spectral bump detector:

Code computes a running median. A small window slides across the data, and at each step it picks the middle value of the numbers in that window.

psd_segment [-50 -50 -50 -48 -45 -35 -45 -48 -50 -50 -50]

[|-50 -50 -50 -48 -45 | -35 -45 -48 -50 -50 -50]-> median -50

[-50  |-50 -50 -48 -45 -35 | -45 -48 -50 -50 -50]-> median -48

[-50 -50  |-50 -48 -45  -35 -45 |-48 -50 -50 -50]-> median -45

[-50 -50 -50 |-48 -45 -35 -45 -48 |-50 -50 -50]-> median -35

This produces the baseline shape of the carrier what the carrier SHOULD look like(envelop)

The np.median arranges the psd_segments and creates envelop that represent how the original carrier should look like

Then residual = original psd- envelop gives carrier on top of carrier.

Eg

PSD

[-50 -48 -45 -35 -45 -48 -50]

Envelope

[-49 -47 -45 -45 -45 -47 -49]

Residual

[-1  -1   0  +10   0  -1  -1] +10db might be interference

Then do threshold check

If residual>threshold then flag it

Now identify the start bin and end bin where that signal exists and highlight them.

A minimum width filter is used to ignore random noise spike to be detected. If only one FFT bin spikes, it is ignored

- Local Variance Detector

Instead of looking for a single spike, it checks how much the PSD fluctuates in a small window. If the fluctuations are too large, it means another signal or interference may be present inside the carrier.

psd_segment: [-2  -1   0   1   0  -1  -2]

[|-2 -1 0 1 0| -1 -2]      → variance ≈ small

[-2 |-1 0 1 0 -1| -2]      → variance ≈ small

[-2 -1 |0 1 0 -1 -2| ]     → variance ≈ small

psd_segment with interference: [-2  -1   0   6   12   6   0  -1  -2]

[|-2 -1 0 6 12| 6 0 -1 -2]   → variance large

[-2 |-1 0 6 12 6| 0 -1 -2]   → variance large

[-2 -1 |0 6 12 6 0| -1 -2]   → variance large

The algorithm slides a window across the carrier and calculates local variance cantered at each bin.

Step 1 — Mean

mean = (-2 -1 + 0 + 6 + 12) / 5

mean = 15 / 5

mean = 3

Step 2 — Difference from mean

-2 - 3 = -5

-1 - 3 = -4

0 - 3 = -3

6 - 3 =  3

12 - 3 =  9

Step 3 — Square the differences

(-5)² = 25

(-4)² = 16

(-3)² = 9

3²   = 9

9²   = 81

Step 4 — Average

variance = (25 + 16 + 9 + 9 + 81) / 5

variance = 140 / 5

variance = 28

Result

local variance = 28 give as image chart

The algorithm compares each bin's variance with a threshold. The algorithm groups adjacent flagged bins into a region. After grouping, the code checks width.

the algorithm uses a small neighbourhood of bins around the current bin to measure how much the signal fluctuates. By looking at several neighbouring bins (the window), the algorithm can measure how much the power changes around that point. If the neighbouring bins vary a lot, the variance becomes large, indicating a possible interference or hidden carrier inside the main carrier.

- Curvature:

First the psd is smoothed to remove noise.

Then slope is calculated sm

6   15   17   15   6

15 - 6  = 9

17 - 15 = 2

15 - 17 = -2

6 - 15  = -9

Then it becomes 9 2 -2 9

Then curvature is calculated

2 - 9   = -7

-2 - 2  = -4

-9 - (-2) = -7

Interpretation

If the spectrum is smooth:

sm
2   3   4   5   6

First derivative:

1 1 1 1

Second derivative:

0 0 0

So curvature ≈ 0.

If a sharp peak appears:

sm
2   4   10   4   2

First derivative:

2 6 -6 -2

Second derivative:

4  -12  4

Curvature becomes large, indicating a sharp spectral peak.