- import zmq- zmq acts as link or pipeline that delivers live RF data packets without delays. zmq (ZeroMQ) is a high-performance messaging library used for real-time communication between processes or systems. It’s much faster and lighter than traditional sockets. It sends and receives data streams, Handles asynchronous communication, Works across threads, processes, or even machines.

- import numpy as np- numpy is the core Python library for Arrays Mathematical operations and Signal processing. It handles large datasets efficiently, Performs vectorized operations (very fast), Supports complex numbers (important for IQ signals)

- import matplotlib.pyplot as plt- This is a plotting library used to plot graphs, charts and real time plots. Draws frequency vs power graphs, Updates plots dynamically and Supports interactive UI elements.

- import threading-Python module for running multiple tasks simultaneously. This helps to run background processes, keeps UI responsive, Handles real time data fetching.

- pyQt5- PyQt5 is a Python library used to create desktop applications with buttons, windows, text boxes, layouts, etc.

  - QtCore- It is the brain of PyQt5. It handles the logic behind the UI, Event handling (clicks, signals), Timing, threading, alignment

  - QVBoxLayout — Vertical Arrangement(arranges elements top to bottom)

  - QHBoxLayout — Horizontal Arrangement(arranges elements left to right)

  - QTextEdit- The place where one can edit or display the texts

  - QPushButton- this is a clickable button that trigger an event upon getting clicked

  - QLabel- Displays the text as labels

  - QApplication-The main engine that runs your GUI. Starts the app, Handles events (clicks, updates)

  - Qt- Qt is a framework that helps you build graphical applications with windows, buttons, and user interfaces across platforms.

- from scipy.signal import get_window, find_peaks:

  - get_window-Applies a window function (like Hann) to smooth the edges of your signal before doing FFT. Without it, FFT causes spectral leakage (fake spreading of energy across frequencies).

  - find_peaks- Detects peaks (high points) in your signal array. It is needed to identify carriers, interference, valleys etc. Locates signal centres, Helps detect interference inside carriers

- from collections import deque- deque (pronounced deck) is a double-ended queue from Python’s collections module. You can add/remove items from both ends very fast. In my code This creates a fixed-size memory buffer. Stores recent signal strengths (last N frames). Automatically removes old values when new ones come. deque is a rolling memory buffer that keeps only the latest data to help your algorithm decide if a signal is stable or not.

- FFT_SIZE: Controls frequency resolution. Higher FFT_SIZE → finer resolution (better detail), Lower FFT_SIZE → faster but blurry. The Fast Fourier Transform (FFT) is an efficient algorithm that converts a time-domain signal into its frequency-domain components (spectrum), breaking it down into individual sine/cosine waves. FFT size is critical because it dictates frequency resolution and computational speed, balancing the trade-off between seeing distinct frequency peaks and tracking rapid signal changes.

- Hardware Sample Rate is the ADC sampling rate of the SDR front-end. If: HW_SAMPLE_RATE = 20e6 Then the SDR captures: ±10 MHz around the hardware centre frequency.

- Display Bandwidth is purely a visual scaling parameter. It defines: X-axis span in the GUI, Frequency labelling limits and Zoom window for plotting. It does not change: Actual sampled data, FFT bin spacing, Detection logic. If DISPLAY_BW = 20e6 You are just telling Matplotlib, Plot ±10 MHz on the x-axis, even if hardware sampled 20 MHz or 40 MHz

DISPLAY_BW != HW_SAMPLE_RATE

- Hardware Centre Frequency is the actual LO tuning frequency of the SDR i.e. SDR LO tuned to 70 MHz Spectrum spans 70MHz±10 MHz. It Display Centre Frequency is  a visual offset only You want to shift spectrum in GUI, You are digitally frequency shifting data, You want logical relabelling. It does NOT retune the SDR.

ZMQ_ADDR = "tcp://127.0.0.1:5555"- This is the main data pipeline of your system. It tells your program where to listen for incoming IQ (signal) data using ZeroMQ. The tcp:// means it’s using network communication, and 127.0.0.1 means “this same computer” (localhost). Port 5555 is simply the channel number. So effectively, your SDR or upstream process is publishing live signal samples, and your code is subscribing to them here. If this address is wrong, your analyzer becomes blind because no signal data will arrive. If you change it to another IP, your program can receive data from a different machine. This line directly affects whether your system gets real-time RF input or nothing at all.

ZMQ_META_ADDR = "tcp://127.0.0.1:5556"- This pipeline carries metadata such as sample rate, FFT size, and center frequency. Think of it as the “settings stream” that tells your analyzer how to interpret the incoming signal. For example, if FFT size or sampling rate changes dynamically, this metadata ensures your program updates accordingly. Without this, your FFT scaling, frequency axis, and detection logic could become incorrect even if data is coming in. So this line ensures your analyzer is not just receiving data, but also understanding how that data is structured.

SMOOTH_BW_HZ Controls how much the spectrum is averaged across nearby frequency bins (~5 bins here). Higher smoothing → cleaner but less detailed (may hide narrow signals).
Lower smoothing → noisy but preserves sharp peaks.

MIN_CARRIER_BW_HZ- Defines the minimum width a signal must have to be treated as a valid carrier (~5 bins). Filters out narrow noise spikes and false detections.
Too high → miss real narrow signals; too low → detect noise as carriers.

THRESHOLD_RATIO

Sets detection threshold between noise and peak: noise + ratio × (peak - noise).
Lower value → more sensitive (more detections, more false alarms).
Higher value → stricter (fewer detections, may miss weak signals).

# =========================

# ZMQ SETUP

# =========================

ctx = zmq.Context()

sock = ctx.socket(zmq.SUB)

sock.connect(ZMQ_ADDR)

sock.setsockopt(zmq.SUBSCRIBE, b"")

sock.setsockopt(zmq.CONFLATE, 1)

sock.setsockopt(zmq.RCVHWM, 1)

sock.setsockopt(zmq.LINGER, 0)

carrier_sock = ctx.socket(zmq.SUB)

carrier_sock.connect("tcp://127.0.0.1:5557")

carrier_sock.setsockopt(zmq.SUBSCRIBE, b"")

carrier_sock.setsockopt(zmq.CONFLATE, 1)

carrier_sock.setsockopt(zmq.RCVHWM, 1)

meta_sock = ctx.socket(zmq.SUB)

meta_sock.connect(ZMQ_META_ADDR)

meta_sock.setsockopt(zmq.SUBSCRIBE, b"")

meta_sock.setsockopt(zmq.CONFLATE, 1)

meta_sock.setsockopt(zmq.RCVHWM, 1)

# =========================

# SHARED STATE

# =========================

_state_lock = threading.Lock()

_latest_iq        = None

_latest_meta      = None

_latest_carriers  = None

This code sets up your program like a live control room that listens to three different radio channels at the same time and safely stores the latest information.

First, zmq.Context() creates the communication environment, like opening a communication system. Then three “ears” (called subscriber sockets using zmq.SUB) are created: sock connects to the main address to receive raw signal data (IQ samples), carrier_sock connects to another port to receive detected carrier information, and meta_sock connects to get metadata like sample rate and FFT size.

Each socket uses setsockopt(zmq.SUBSCRIBE, b"") which means “listen to everything coming in.” The options CONFLATE = 1 and RCVHWM = 1 ensure that only the latest data is kept and older data is discarded, which is very important in real-time systems so the display doesn’t lag behind. LINGER = 0 ensures the socket closes immediately without waiting. After setting up these live data streams, the code creates a thread lock (_state_lock) which acts like a safety lock on a shared notebook, so that only one part of the program writes or reads data at a time, preventing corruption.

Finally, _latest_iq, _latest_meta, and _latest_carriers are variables that store the most recent signal data, metadata, and carrier information respectively. So overall, this block builds a real-time, low-delay data receiving system that always keeps the newest information and safely shares it across the program without conflicts.

class DataFetcher(threading.Thread):

def __init__(self):

super().__init__(daemon=True)

self._stop_event = threading.Event()

self._poller = zmq.Poller()

self._poller.register(sock,         zmq.POLLIN)

self._poller.register(meta_sock,    zmq.POLLIN)

self._poller.register(carrier_sock, zmq.POLLIN)

def stop(self):

self._stop_event.set()

def run(self):

global _latest_iq, _latest_meta, _latest_carriers

while not self._stop_event.is_set():

events = dict(self._poller.poll(timeout=10))

if not events: continue

if sock in events:

data = sock.recv()

iq = np.frombuffer(data, dtype=np.complex64)

with _state_lock: _latest_iq = iq.copy()

if meta_sock in events:

with _state_lock: _latest_meta = meta_sock.recv_json()

if carrier_sock in events:

with _state_lock: _latest_carriers = carrier_sock.recv_json()

This piece of code creates a background worker that continuously listens for incoming data and updates your program with the latest information in real time. The DataFetcher class is a thread (a parallel task) that runs independently from your main program so your UI doesn’t freeze. When it starts, it sets up a Poller, which is like a smart listener that watches all three sockets (sock, meta_sock, and carrier_sock) at once and tells you when any of them receives data. Inside the run() loop, it keeps checking for new messages until a stop signal is given. If new IQ signal data arrives on sock, it receives the raw bytes, converts them into complex signal values using NumPy, and safely stores them in _latest_iq. If metadata arrives, it stores it in _latest_meta, and if carrier information arrives, it stores it in _latest_carriers. The with _state_lock ensures that this updating happens safely without conflicting with other parts of the program that might read the same data. The stop() function simply tells the thread to exit cleanly. So overall, this class acts like a real-time data collector that always grabs the newest signal, settings, and detection results and keeps them ready for processing and display.

# ═════════════════════════════════════════════════════

# DETECTION TRACKER — Temporal stability, debounce, hysteresis, confidence

# ══════════════════════════════════════════════════════

class DetectionTracker:

"""

Tracks detections (interference, gaps, valleys) across frames using

frequency-bucket matching.  Provides:

• Debounce:   detection must persist N consecutive frames before logging

• Hysteresis: separate ON/OFF thresholds to avoid flicker

• Confidence: rolling score based on persistence + strength stability

• Tracking ID: stable ID per detection across frames

"""

def __init__(self,

bucket_hz=STABILITY_BUCKET_HZ,

debounce_on=STABILITY_DEBOUNCE_ON,

debounce_off=STABILITY_DEBOUNCE_OFF,

confidence_min=STABILITY_CONFIDENCE_MIN,

strength_history_len=STABILITY_STRENGTH_HISTORY,

hysteresis_db=STABILITY_HYSTERESIS_DB):

self.bucket_hz = bucket_hz

self.debounce_on = debounce_on

self.debounce_off = debounce_off

self.confidence_min = confidence_min

self.strength_history_len = strength_history_len

self.hysteresis_db = hysteresis_db

self._tracks = {}      # bucket_key → track dict

self._next_id = 1

self._frame = 0

def _bucket_key(self, freq_hz):

return int(round(freq_hz / self.bucket_hz))

def _new_track(self, freq_hz, strength_db):

tid = self._next_id

self._next_id += 1

return {

'id':           tid,

'center_freq':  freq_hz,

'strength_db':  strength_db,

'first_seen':   self._frame,

'last_seen':    self._frame,

'consecutive':  1,

'absent_count': 0,

'confirmed':    False,

'logged_once':  False,     # True after first log emission

'strength_hist': deque(maxlen=self.strength_history_len),

}

def _compute_confidence(self, track):

"""

Confidence = weighted combination of:

• persistence ratio (consecutive / debounce_on)  — capped at 1.0

• strength stability (1 - cv of recent strengths) — higher = more stable

"""

pers = min(1.0, track['consecutive'] / max(1, self.debounce_on))

hist = track['strength_hist']

if len(hist) >= 2:

arr = np.array(hist)

mu = np.mean(arr)

cv = np.std(arr) / max(abs(mu), 0.01)

stab = max(0.0, 1.0 - cv)

else:

stab = 0.5

return 0.65 * pers + 0.35 * stab

def update(self, detections):

"""

Process one frame of detections.

Parameters

----------

detections : list of dict

Each dict MUST have 'center_freq' (Hz) and 'strength_db'.

May contain any additional fields — they are passed through.

Returns

-------

list of dict

Only confirmed detections (passed debounce + confidence).

Each dict has the original fields PLUS:

'track_id', 'confidence', 'is_new' (True on first confirmation)

"""

self._frame += 1

seen_buckets = set()

confirmed = []

for det in detections:

bk = self._bucket_key(det['center_freq'])

seen_buckets.add(bk)

if bk in self._tracks:

trk = self._tracks[bk]

trk['last_seen'] = self._frame

trk['consecutive'] += 1

trk['absent_count'] = 0

trk['center_freq'] = det['center_freq']

trk['strength_db'] = det['strength_db']

trk['strength_hist'].append(det['strength_db'])

else:

trk = self._new_track(det['center_freq'], det['strength_db'])

trk['strength_hist'].append(det['strength_db'])

self._tracks[bk] = trk

# Check debounce + confidence

conf = self._compute_confidence(trk)

if trk['consecutive'] >= self.debounce_on and conf >= self.confidence_min:

is_new = not trk['confirmed']

trk['confirmed'] = True

out = dict(det)

out['track_id'] = trk['id']

out['confidence'] = conf

out['is_new'] = is_new

confirmed.append(out)

# Age out absent tracks (hysteresis — keep for debounce_off frames)

for bk in list(self._tracks):

if bk not in seen_buckets:

trk = self._tracks[bk]

trk['consecutive'] = 0

trk['absent_count'] += 1

if trk['absent_count'] >= self.debounce_off:

del self._tracks[bk]

elif trk['confirmed']:

trk['confirmed'] = False

return confirmed

def reset(self):

"""Clear all tracking state (e.g. on meta update)."""

self._tracks.clear()

self._frame = 0

# Global tracker instances — one per detection category

_intf_tracker    = DetectionTracker()

_gap_tracker     = DetectionTracker()

_valley_tracker  = DetectionTracker(debounce_on=4)   # valleys need more stability

_carrier_tracker = DetectionTracker(debounce_on=2, debounce_off=8)

This code defines a class called DetectionTracker, which acts like a memory and decision system for your detector so it doesn’t react to noise or flicker but instead tracks real signals over time. It is necessary because your RF system works on continuous frames of data, and without this logic, detections would appear and disappear randomly due to noise, causing unstable results. The class starts with __init__, where parameters like bucket_hz (groups nearby frequencies into buckets), debounce_on (how many consecutive frames a detection must appear before confirming), debounce_off (how many frames it must disappear before removing), confidence_min (minimum reliability score), and strength_history_len (how many past values to store using deque) are initialized; these directly control stability, sensitivity, and responsiveness of detection. The dictionary self._tracks stores all active detections, _next_id gives each detection a unique ID, and _frame counts time steps. The function _bucket_key() groups frequencies into bins so small frequency shifts don’t create new detections unnecessarily. The _new_track() function creates a new detection record with parameters like frequency, strength, counters, and a rolling history buffer. The _compute_confidence() function calculates how reliable a detection is by combining how long it has persisted (consecutive/debounce_on) and how stable its strength is (low variation = high confidence), ensuring only consistent signals are trusted. The main logic is in update(detections), which is called every frame in your processing pipeline (after detection algorithms run); it updates existing tracks or creates new ones, increments counters, stores strength history, and checks if a detection passes both debounce (time stability) and confidence (quality) thresholds. Only such confirmed detections are returned, each with added fields like track_id, confidence, and is_new. Then, the hysteresis logic removes detections slowly: if a signal disappears, it is not immediately deleted but allowed to persist for debounce_off frames to avoid flickering. The reset() function clears all tracking when system parameters change. Finally, multiple instances (_intf_tracker, _gap_tracker, _valley_tracker, _carrier_tracker) are created for different detection types, each with slightly different stability settings. This code comes into effect after your detection algorithms (carrier, interference, valley detection) and directly affects how results are filtered before logging or display. For example, if your raw detection output is: [{"center_freq": 70e6, "strength_db": 12}] appearing intermittently, without this tracker it would flicker ON/OFF, but with it, only after (say) 3 continuous frames it becomes: {"center_freq": 70e6, "strength_db": 12, "track_id": 1, "confidence": 0.82, "is_new": True}, and remains stable even with minor fluctuations. Its importance is critical because it converts raw, noisy detections into stable, reliable, trackable events, ensuring your system behaves like a professional analyzer rather than a noisy detector.