Simple Summary – What Was Causing the Lag?

🔴 Old Design (Lagging Version)

- ZMQ polling, FFT processing, carrier detection, logging, and plotting were all done inside the same update loop.

- If rendering or detection took longer than incoming data rate, messages started queuing.

- Even with CONFLATE=1, polling inside the render loop created timing dependency.

- The GUI waited for data, and data waited for GUI → sync delay.

- This created visible lag between real signal and displayed spectrum.

In simple words:

The GUI was trying to drive the SDR instead of just showing the latest data.

🟢 New Design (Lag-Free Version)

- A dedicated DataFetcher thread continuously reads ZMQ sockets.

- It always overwrites shared variables with the latest frame.

- No queue. No accumulation. Only the newest data survives.

- The GUI render loop (QTimer) just reads whatever is latest and draws.

- If GUI is busy, frames are skipped — never queued.

In simple words:

The SDR runs freely. The GUI only paints the newest snapshot.

🔥 What Actually Fixed the Sync Issue?

- Added CONFLATE=1 to all sockets (IQ, metadata, carrier).

- Moved ZMQ polling to a separate background thread.

- Used shared state with overwrite (not append).

- Render loop reads-and-clears shared data atomically.

- Switched from FuncAnimation to QTimer.

- Used draw_idle() instead of blocking draw().

This removed the backlog completely.

| Feature | Old Code | New Code |
| --- | --- | --- |
| ZMQ Polling | Inside render loop | Background thread |
| Data Handling | Process every frame | Keep only latest |
| Frame Backlog | Possible | Impossible |
| GUI Blocking | Yes (tight coupling) | No (decoupled) |
| Metadata Delay | Could queue | Always latest |
| Carrier Overlay Delay | Could lag | Always fresh |
| Rendering Trigger | FuncAnimation | QTimer |
| Draw Method | Implicit redraw | draw_idle() |
| CPU Efficiency | Medium–High | Controlled & stable |
| Sync Accuracy | Drift under load | Real-time aligned |

NEW: DataFetcher Thread Added

🔹 Added Class (NEW BLOCK)

class DataFetcher(threading.Thread):

This entire class did not exist before.

This block is where ZMQ polling was moved.

Inside it:

events = dict(self._poller.poll(timeout=10))

This is the new polling location.

🟢 3️⃣ NEW: Shared State Variables Added

This block is completely new:

_state_lock = threading.Lock()

_latest_iq        = None
_latest_meta      = None
_latest_carriers  = None

Old code did not have shared-state storage.

🟢 4️⃣ NEW: Atomic Snapshot in update()

At the beginning of update():

with _state_lock:
    iq_frame       = _latest_iq
    meta_frame     = _latest_meta
    carrier_frame  = _latest_carriers
    _latest_iq       = None
    _latest_meta     = None
    _latest_carriers = None

This entire block is new.

This is what prevents backlog.

🟢 5️⃣ Render Engine Changed

❌ OLD

ani = FuncAnimation(fig, update, interval=40)

✅ NEW

_render_timer = QtCore.QTimer()
_render_timer.timeout.connect(update)
_render_timer.start(33)

You replaced Matplotlib animation with Qt event-loop timer.

🟢 6️⃣ draw_idle() Added

At the bottom of update():

fig.canvas.draw_idle()

This is also new.

Old version forced redraw each frame.

Summary of all namespace and design decisions

Why psd_s is computed only once: both detectors use the same SMOOTH_BW_HZ convolution. Doing it twice would be the only meaningful CPU cost — avoided entirely.

Why carrier_threshold and intf_threshold are separate: despite sharing psd_s, the sensitivity levels differ (0.35 vs 0.30). Sharing a single threshold would make the interference detector silently use carrier-level sensitivity.

Why zorder is layered (2 → 7): carrier green spans sit at zorder=2, interference red spans at zorder=5. This means a frequency that triggers both detectors shows the interference highlight visually on top of the carrier highlight — which is the physically correct interpretation since interference is riding on top of a carrier.

Why interference labels are placed at Y_MAX - 15: carrier labels use Y_MAX - 5. The 10 dB vertical offset prevents label collision when both detectors fire on the same frequency region.

Variable renaming summary:

| Old (collision risk) | New carrier name | New interference name |
| --- | --- | --- |
| rises | carrier_rises | intf_rises |
| falls | carrier_falls | intf_falls |
| threshold | carrier_threshold | intf_threshold |
| carrier_id | carrier_count | intf_count |
| color (log) | GREEN_SHADES[...] | RED_SHADES[...] |